Versie: 28 juni 2024

BRISTELL BR23         CHECKLIST            1

Before Engine Start

Preflight/Daily inspection	COMPLETED

Passenger briefing………  COMPLETED

Papers / Map………….......	CHECKED/ON BOARD

Loose objects…………….	SECURED

Circuit breakers …….……  CHECK ALL IN

Master Switch …………….	ON

Backup Battery……………	ON

EFIS switch………………..  ON

Seat Belts	…………………	FASTEN

Rudder pedals………….…	ADJUSTED/SECURED

Flight Controls ……….…..	FULL / FREE AND CORRECT

Trims  ……………………… SET elevator and aileron

Fuel Caps………………….  SECURED

Wing Lockers……………..  CLOSED & LOCKED

Canopy……………………..	CLOSED & LOCKED

Parking brake……………..  ON

Altimeter …………………...	SET PFD

PFD…………………………. Set split (PFD and EMS)

Fuel quantity……………… CHECK/SET ON EMS

Weight & Balance………..  SET/CHECK Limits

Versie: 28 juni 2024

2       BRISTELL BR23                CHECKLIST

Engine Start

Fuel selector………………  LEFT TANK

Fuel pump ………………… ON

Choke ……………………...	AS NEEDED

Prop control……. ………… FULL FORWARD

Throttle …………………….	JUST OPEN

Strobe Light……………….  ON

Start Engine ………………	TURN KEY TO START

Oil Pressure ………………	CHECK 2-7 BAR

Engine RPM ………….…… 2000 RPM FOR                                                                                                                                  	                                     WARMING UP (OIL < 50°C)

Choke ……………………...	GRADUALLY RELEASE

Before Taxiing

After Engine Start

Backup Battery……………	OFF

Fuel pump …………………	OFF

Avionic switch	 ………….	ON Check minimum 11.0V

ALT-1 switch……………...  ON Check increase to 13.4V

ALT-2 switch……………...  ON Check increase to 13.8V

Engine Limits:

Speed 			5500 - 5800 rpm (5 min.)

5500 rpm (continuous)

Cyl. Head Temp.		90° – 135° C (135°C max.)

Oil Temp.			90°C - 110°C (130°C max.)

Exh. Gas Temp.		880°C max..

Oil Pressure			2 - 5 bar (7 bar max. cold eng.)

Fuel Pressure		0,15 – 0,4 bar

Versie: 28 juni 2024

BRISTELL BR23         CHECKLIST            3

Before Taxiing

Cockpit lights……………..	AS REQUIRED

Nav lights …………………	ON

Parachute …………………  SAFETY PIN REMOVED

Wind direction …………...  CHECK (For runway in use)

Transponder………………  Set to 7000

Oil Temp …………………..  MIN. 30°C

Time ………………………..  CHECK (Start flight timing)

Radio call …………………	APPLY

Parking Brake ……………	RELEASE / CHECK BRAKES

Versie: 28 juni 2024

4       BRISTELL BR23                CHECKLIST

ENGINE RUN-UP

Parking brake …………….	ON

Fuel selector ……………..	RIGHT TANK

Engine instruments ……..	CHECK, WITHIN LIMITS

Prop control……. ………… FULL FORWARD

Ignition Check ………….... 	THROTTLE 4000 RPM

Test Ignition ………….…..	Drop MAX.300 diff 115 rpm

Carburetor Heat …….…...	PULL HOT, Check drop MIN. 150 RPM, PUSH OFF

Prop control……………….  CYCLE 2 times at 4500 RPM, Check drop > 200 rpm

Prop control……..………… FULL FORWARD

Engine instruments ……..	CHECK OIL & FUEL PRESS

Throttle ………………….…	IDLE (1700-2000 RPM)

Fuel Selector……………… LEFT TANK

Fuel pump…………………  ON, check pressure

BEFORE TAKEOFF

Flaps ………………….…… SET 10° (stage 1)

Altimeter ……….…............ CHECK BOTH

Instruments ……………....  CHECK WITHIN LIMITS

Canopy …………………..... CHECK CLOSED & LOCKED

Seat Belts	…………………	FASTEN

Landing Light…………….   ON

PFD…………………………  FULL SCREEN

Versie: 28 juni 2024

BRISTELL BR23         CHECKLIST              5

TAKEOFF

Wind……………………...… CHECK

Brakes……………………… RELEASE

Throttle …………………….	MAX (Min. 5500 RPM)

Airspeed indication……… CHECK “ALIVE”

Initial climb………………..  64 KIAS (Vx)

CLIMB

> 100 ft:  Prop control…..  < 5500 RPM 
> 150 ft:  Flaps……………  UP

Climb speed ……………… 74 KIAS

Level off
Manifold Pressure…….…  26-26,5 Inch

Prop control………………  4800-4850 RPM
Fuel pump………………….	OFF

Engine Instruments ……..	CHECK WITHIN LIMITS

APPROACH

Brakes ……………………..	OFF / CHECK PRESSURE Seat belts ……………….…	FASTEN

Autopilot…………………… DISENGAGE

PFD …………………………	FULL SCREEN

Engine Instruments ……..  CHECK WITHIN LIMITS

Landing Light …………….	ON

Fuel Pump ………………...	ON check pressure and QTY

Carburetor heat .…………	PULL HOT (if applicable)

Throttle…………………….  21-15 Inch

Prop control………………  4800-4850 RPM 
Fuel Selector………………	FULLEST TANK

Versie: 28 juni 2024

6       BRISTELL BR23                CHECKLIST

LANDING

Airspeed…………………… 75 KIAS

Flaps ……………………….	SET 10° (stage 1)

Airspeed…………….….….	70 KIAS

Airspeed…………….….….	65 KIAS

Flaps ……………………….	SET 25° (stage 2)

Airspeed…………………… 60 KIAS

On Final: Propeller…,,,,.… FULL FORWARD

AFTER LANDING

Throttle…………………….. As required for taxi

Carburetor Heat ..……….	OFF

Flaps ……………………….	UP

Fuel Pump ………………...	OFF

Landing Light …………….	As required

AIRCRAFT PARKING

Parking Brake …………….	SET

Prop control……………….. CHECK FULL FORWARD

Throttle ……………………	IDLE

Trims ……………………….	SET TO NEUTRAL

Instruments ………………	CHECK WITHIN LIMITS

Time ……………………….   CHECK (Stop flight timing)

Ignition Switch …………..	TURN KEY OFF & REMOVE

Versie: 28 juni 2024

BRISTELL BR23         CHECKLIST               7

Lights………………………  OFF

EFIS…..…………………….  OFF

Avionics Switch …..……..	OFF

Alt-2 and Alt-1…………….  OFF

Master switch…………….  OFF

Fuel Selector……………..  OFF

Parachute ………………… SAFETY PIN IN

Performance data for variable pitch propellor

Versie: 28 juni 2024

8       BRISTELL BR23                CHECKLIST

AIRCRAFT LIMITS

VNE never exceed	   		157 KTS

VA maneuvering			  99 KTS

Vno Max cruising speed		136 KTS

Vfe						  82 KTS

VX best angle climb		  62 KTS

VY best rate climb			  74 KTS

Cruise climb				  75 KTS

Vs1 stall clean				  51 KTS

Vso full flaps				  44 KTS

Best glide speed			  67 KTS

Max. dem. crosswind		  15 KTS

G-limits flaps up			  +4 / -2

G-limits flaps down		  +2 /  0

Empty weight					448 	   KG

Max baggage weight wings       2x20      KG	Max baggage behind the seats   	  28     KG

Max. T/O weight				750	   KG

Max Forward CG Limit			25%    MAC

Max AFT CG Limit				31%    MAC

Bristell EMERGENCY Procedures

ENGINE FAILURE DURING T/O RUN                .

Throttle ………………….. IDLE

Brakes …………………... APPLY

Ignition ………………….. OFF

ENGINE FAILURE AFTER TAKEOFF              .

Airspeed ………………… 67 KNOTS

Flaps …………………….. AS NECESSARY

Fuel Selector …………... OFF

Ignition Switch ………… OFF

Master switch ………….. OFF

Land straight ahead, turning only to avoid obstacles

LOSS OF ENGINE POWER IN FLIGHT            .

Speed ………………..…..	67 KIAS

Inflight Engine Restart …	APPLY

OR PERFORM  EMERGENCY  LANDING

EMERGENCY DESCENT                      .

Airspeed ………………… MAX PERMITTED:

VNE  = 157 KIAS

Vno  = 136 KIAS

VA   =   99 KIAS

Engine RPM …………….   MAX. 5800 RPM

Bristell EMERGENCY Procedures

ENGINE FIRE DURING T/O RUN                 .

Throttle ……………...	IDLE

Fuel Selector ……….	OFF

Fuel Pump ………….	OFF

Cabin Heating ………	OFF

Brakes ………………	APPLY AS NEEDED

When airplane is under control:

Ignition ………………	TURN KEY OFF

Master ……………. …OFF

Parking Brake ………	SET

Evacuate the aircraft!

ENGINE FIRE IN-FLIGHT                   .

Cabin Heat …………	PUSH OFF

Fuel Selector ………	OFF

Throttle ……………..	MAX

Ignition Switch …….	OFF AFTER ENGINE STOPS

Airspeed ……………	EMERGENGY DECENT

Master Switch ……..	OFF

Perform emergency landing!

Evacuate the aircraft!

COCKPIT  FIRE		………………….

Master Switch ……..	OFF

Cabin Heating ………	OFF

Ventilation ………….	OPEN

PERFORM EMERGENCY LANDING  AS SOON  AS POSSIBLE

Bristell EMERGENCY Procedures

IN-FLIGHT  ENGINE  RESTART                   .

Airspeed…………………… 75 KIAS

Fuel Pump ………………...	ON

Fuel Selector ……………..  FULLEST  TANK

Throttle …………………….	JUST OPEN

Carburetor Heat …….…...	PULL HOT

Ignition Switch ……………	TURN KEY

After engine is running

Engine instruments ……..	CHECK, WITHIN LIMITS

Avionics …………………..	ON

Fuel Pump ………………..	OFF

EMERGENCY LANDING  WITHOUT  ENGINE  POWER

Airspeed ………………..….. 67 KIAS

Locate suitable terrain without obstacles

Propeller control ………… FINE PITCH (fully Forward)

Trim…………………………. Adjust

Seat Belts	…………………	Tighten

Landing Light…………….   ON

Flaps ………………………	Extend as needed

Radio call …………………	Apply (MAYDAY)

Transponder ……………...	Set 7700

Approach …………...........	WITHOUT STEEP TURNS

Fuel Selector……………...  OFF

Before Touchdown

Master Switch………... 	OFF

Bristell EMERGENCY Procedures

POWER-ON  FORCED  LANDING                    .

A precautionary landing is generally carried out in the cases where the pilot may be disorientated, the aircraft has no fuel reserve or possibly in bad weather conditions.

1. Choose landing area, determine wind direction.

2. Report your intention to land and landing area location.

3. Perform low-altitude passage into wind over the right-hand side of the

chosen area with flaps extended as needed and thoroughly inspect the

landing area.

4. Perform circle pattern.

5. Seat belts  - fasten

6. Perform approach at increased idling with flaps in landing position (25°) at

67 KIAS.

7. Reduce power to idle when flying over the runway threshold and

touch-down at the very beginning of the chosen area.

8. After stopping the airplane:

Ignition Switch……………………. TURN KEY OFF & REMOVE

ALL Switches……………………... OFF

FUEL selector……………………..  OFF

Airplane ……………………………. lock and seek assistance

RECOVERY FROM UNINTENTIONAL SPIN          .

Power ……………….	IDLE

Flaps ………………..	RETRACT

Ailerons …………….	NEUTRAL

Rudder ……………..	FULL OPPOSITE

Elevator …………….	PUSH FORWARD

HOLD THESE INPUTS UNTIL THE ROTATION STOPS THEN:

Rudder ………………	NEUTRAL

Elevator ……………..	PULL GENTLY TO

RECOVER FROM DIVE

Bristell Pre-flight Inspection

1)	Ignition ………………….…..	OFF

Avionics/Instruments ………	Check condition

Cockpit ………………………	Check for loose items and condition

Master switch ………………	ON

Avionics switch………………	ON

Fuel quantity indication …….  Compare visual with indication

Flaps …………..……………..  Check operation

Lights …………………………	Check operation

Pitot tube heating ……………	Check function

All switches  ………………….	OFF

Control System………………  Check function, free movement

Canopy………………………..  Condition of attachment, cleanness

Bristell Pre-flight Inspection

2)	Engine cowling condition

Propeller, blades and spinner condition

Oil quantity check

Coolant quantity check

Engine mount and exhaust manifold condition

Inlets free of obstruction

Fuel system draining / gascollator

Nose gear condition

3)	Wing surface condition

Leading edge condition

Stall strips condition

Wing locker, Closed and locked

Pitot free of obstruction

Fuel vent openings

4)	Wing tip, surface condition, attachment

Lichts, condition attachment

Aileron, surface condition, attachment

Flap, surface condition, attachment

Wing locker, Closed and locked

Fuel caps, in place en closing

Pitot free of obstruction

Fuel vent openings

5)	Landing gear, check for condition and damage

Weel attachment, brakes condition and pressure tires

Wing lower surface and fuselage bottom surface condition

Static port free of obstructions

6)	- Vertical Tail Unit Condition of Surface, Attachment free rudder       Movement, (press tail down to have nosegear free) antenna.

- Horizontal Tail Unit Condition of surface, Attachment, free      elevator movement elevator tips and mass balance.

- Check trim/anti-servo tab

- Check free movement of tab for full elevator movement